FORM ONE WOODWORK MARKING SCHEME

- a) Name three types of attires used in the workshop

- Apron

- Overall

- Helmet

- Boots

- Gloves

- Caps

- Goggles

3x1=3mks

b) State three safety measures in the relation to sharp edged tool.

- Should be carried close to the body with sharp edge pointing downwards

- When handing over, the receiver should pick it from the handle side

- Should not be stored in metallic boxes as they would easily get blunt

- Should be used to operate away from your body

- Should be sharpened when blunt

3x1=3mks

- Name five measuring tools

- Bar rule

- Folding rule

- Calibrated try square

- Calipers

- Tape measures

- Slidding beve

- Collapsible rule

- Zig-zag rule

- Metre rule

3x1=3mks

- Draw a well labeled sketch of a marking gauge	(labeling=5MKS)

Labeling =5mks

- Name the three classifications of saws and give two types of saws under each class.

a) HANDSAWS

- Ripsaw

- Cross-cut saw

b) BACK SAWS

- Tenon saw

- Dovetail saw

c) CURVE CUTTING SAWS

- Coping saw

- Fret saw

- Compass saw

- Bow saw

- Span saw

- Key hole/ pad saw

Naming 3x2=3mks

Types 2x3=6mks

total=9mks

- Name the three bench planes

- Smoothing plane

- Jack plane

- Try plane

3x1=3mk

- (a)Name of the tool

- Jack plane												1mk

b) Name the labeled parts of the tool in (6a) above.

- Lever cap

- Cap iron

- Blade

- Lateral adjustment lever

- Handle

- Frog

6x1=6mks

- Name the three plane adjustments

- Cap iron adjustment

- Depth of cut adjustment

- Lateral blade adjustment

- Frog adjustment

3x1=3mks

- Label the parts shown on the drawing below

- Countersunk hole

- Clearance hole

- Pilot hole

- Work pieces (wood)

- Show sketches of the following timber defects

- Name the three types of dividers

- Spring divider

- Wing divider

- Plain divider

3x1=3mks

- a) What is clogging? 1mk)

- clogging is the accumulation of wood shavings at the mouth of the plane blocking it

1mk

b) State three causes of clogging

- small plane mouth opening

- loose fixing of the blade to cap iron

- planing wood against the grain

- blunt plane blade

3x1=3mks

- Name three methods of planing end grain in wood and show a sketch in each case

Total =9mks

- Draw a well labeled cross-section of a log

Phloem

- Define the following shaping terms

a) INLAYING (2mks)

- It is the method used in decorating furniture by embedding wood or other kind of materials on wood surfaces. (2mks)

b) OVERLAYING

- It is fixing wood or any other material on wood surfaces for decoration purposes. (2mks)

c) CURVING

it is a method of shaping by cutting and removing material on wood along a line that is not straight. 			(2mks)

d) ROUNDING

- It is a method of shaping edges and ends of wood to given radius. 		(2mks)

- Name and show sketches of three methods of nailing

- State three important things mentioned when ordering nails

- Size

- Type

- Quantity

- Quality

3x1=3mks

- State four advantages of screws over

- Screws have grater holding power

- Screws have a neat appearance

- Screws can be removed easily

- When correctly fitted, there is no danger of splitting timber

3x1=3mks

- Show the following types of lines used in drawing work

a

b

c

d

4x1= mks